Equipment Maintenance

SAVAGE BROS FUDGE STOVE

Overview

Care and cleaning is necessary to keep your stove in top operating condition. The complete stove should be given a periodic general cleaning.  Follow the manufacturers recommendations based on the stove you have.

| Task | Who | Frequency |
| --- | --- | --- |
| 1. Cleaning | Owner | Daily |
| 2. Check Jets (gas stove) | Owner | Monthly |
| 3. Clean Jets (gas stove) | Trained Professional Service Technician | As Needed |
| 4. Inspect Element (electric stove) | Owner | Monthly |

Task Procedures

- Daily general cleaning of the stainless steel housing with a soft cloth and non-abrasive cleaner.

- Gas stove - check the flame color to see if there is a wide variation from the jets.  Natural gas flame is high in blue color – Liquid propane flame is high in yellow color.

- Jets are designed to be removed for cleaning the manufacturer recommends this to be done occasionally by a service company or qualified installer based on the color of the flame.

- Inspect the heating element for breaks and burned out sections.  You will want to be careful not to spill foods into the heating elements.  Foods or liquids in contact with the hot element have a tendency to burn the element wire.  If this happens, the only solution is to replace the entire element and insulating dish assembly.